|  | CENTER 303 Thermometer - K-Type J-Type - Dual Input - Dual Display - Simulatneous MAX AVG MIN Measurement - Auto Power Off - REL Function - HOLD Function - Timer Function - Resolution 0.1 °C,0.1°F - External battery access door for easy battery changing Display : 3 1/2 digit LCD Low Battery: symbol will appear when the battery is low. Measurement Range: K-Type -200°C~ 1370°C(-328°F~ 2498℉) J-Type -200°C~ 760°C(-328°F~ 1400°F) Accuracy : -200°C ~ 1370°C…. ±0.1% rdg + 0.7°C -328°F ~ 2498°F…. ±0.1% rdg + 1.4°F Input Protection: 60V DC or 24Vrms AC Maximum Battery Type: 9V, NEDA 1604, IEC 6F22, JIS 006P Operating Temp.: 0°Cto 40°C( < 80% humidity ) Storage Temp.: -10°Cto 60°C( < 70% humidity ) Dimension: 184mm×64mm×30mm( 7.25"×2.5"×1.2") Weight : Approx. 210g Accessory: Manual, 9V battery, Carrying case, K type bead sensor x 2 |
| --- | --- |